# TAITA TAVETA NATIONAL POLYTECHNIC

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

MECHATRONIC TECHNICIAN

SMART HOSPITAL QUEUE MANAGEMENT SYSTEM WITH TRIAGE AND SMS NOTIFICATION

Prepared by: Humphrey Kombe Nzai

Reg No:19596

This project is submitted in partial fulfilment of requirement for the Taita Taveta National Polytechnic award of DIPLOMA OF MECHATRONIC TECHNICIAN

# DECLARATION

I hereby declare that this project report is based on my original work except for citations and quotations which have been duly acknowledged. I also declare that it has not been previously and concurrently submitted for any other degree or award

Signature:_______________

Name: Humphrey Kombe Nzai

ID No:42727481

Date:________________

SUPERVISOR

I the undersigned do hereby certify that this is a true report for the project undertaken by the above named trainee under my supervision and that it has been submitted to Taita Taveta National Polytechnic with my approval

Signature:________________

Date:________________

# DEDICATION

Specially dedicated to

My beloved Mom and Dad, my Uncles and Aunts and to my Siblings

# ACKNOWLEDGEMENT

I would like to express my sincere gratitude to my supervisor Stephanie Masika for the continuous support, guidance, and encouragement throughout the development of this project. I also acknowledge Taita Taveta National Polytechnic for providing the learning environment and technical knowledge necessary for the completion of this project. Finally, I appreciate my colleagues and family members who offered support and motivation during the research and development process.

# ABSTRACT

The increasing patient congestion in hospital outpatient departments has created the need for automated queue management systems that can efficiently control patient flow, prioritize emergency cases, and provide real-time communication to patients. This project presents the design and implementation of a Smart Hospital Queue Management System with triage selection and SMS notification capabilities.

The system is developed using microcontroller-based embedded technology to automate token issuance, symptom-based patient classification, and queue monitoring. Patients interact with the system through a keypad interface where they select their symptom category. The system then generates a queue token, assigns the patient to the appropriate service room, and sends an SMS notification when the patient is two positions away from being served.

The prototype integrates an LCD display, keypad input, buzzer alert system, and GSM communication module to demonstrate real-time queue control. Experimental testing indicates that the system improves service efficiency and reduces waiting time in outpatient departments.

# TABLE OF CONTENTS

Contents

DECLARATION	2

DEDICATION	3

ACKNOWLEDGEMENT	4

ABSTRACT	5

TABLE OF CONTENTS	6

CHAPTER ONE: INTRODUCTION	9

1.1 Background of the Study	9

1.2 Problem Statement	9

1.3 Research Objectives	9

1.4 Research Questions	9

1.5 Significance of the Study	10

1.6 Scope of the Study	10

1.7 Limitations of the Study	10

CHAPTER TWO: LITERATURE REVIEW	11

2.1 Introduction to Literature Review	11

2.2 Concept of Queue Management Systems	11

2.3 Embedded Systems in Healthcare Applications	12

2.4 Triage Systems in Healthcare	12

2.5 SMS Communication in Healthcare Systems	12

2.6 Review of Related Studies	12

2.7 Research Gap	12

CHAPTER THREE:METHODOLOGY	13

3.1 Introduction	13

3.2 Research Design	14

3.3 System Architecture	14

3.4 System Block Diagram	15

3.5 Hardware Components	15

3.5.1 Arduino Uno Microcontroller	15

3.5.2 Matrix Keypad	16

3.5.3 LCD Display (16×2 I2C)	16

3.5.4 Buzzer	16

3.5.5 SIM800L GSM Module	16

3.5.6 Power Supply Unit	17

3.6 Software Development	17

3.7 Triage Classification Algorithm	17

3.8 Data Collection	18

3.9 System Testing	18

3.9.1 Hardware Testing	19

3.9.2 Integration Testing	19

3.9.3 Performance Testing	19

3.10 Ethical Considerations	20

3.11 Chapter Summary	20

CHAPTER FOUR	20

SYSTEM DESIGN, IMPLEMENTATION AND RESULTS	20

4.1 Introduction	20

4.2 System Design Overview	20

4.3 Hardware Design	21

4.3.1 Hardware Components Used	21

4.4 Circuit Design	21

4.4.1 Keypad Connections	21

4.4.2 LCD I2C Connections	22

4.4.3 Buzzer Connection	22

4.4.4 GSM Module Connections	22

4.5 Software Design	23

4.6 Token Generation System	23

4.7 Triage Selection System	24

4.8 SMS Notification System	24

4.10 System Operation	25

4.11 System Testing Results	26

4.12 Challenges Encountered	26

4.13 System Advantages	27

4.14 Chapter Summary	27

CHAPTER FIVE:	28

CONCLUSION AND RECOMMENDATIONS	28

5.1 Introduction	28

5.2 Summary of the Study	28

5.3 Achievement of Project Objectives	29

Objective 1: Design a keypad-based token generation system	29

Objective 2: Implement a triage-based patient classification system	29

Objective 3: Display queue information on an LCD screen	29

Objective 4: Implement SMS notification using a GSM module	30

Objective 5: Provide audio alerts for queue notifications	30

5.4 Conclusion	30

5.5 Recommendations	30

5.6 Suggestions for Further Research	31

5.7 Final Remarks	31

REFERENCE	32

APPENDIX 1:Arduino Code	32

LIST OF FIGURES

FIGURE 2 LCD AND KEYPAD TESTING	18

FIGURE 3 TOKEN GENEREATION TESTING	18

FIGURE 4 GSM TESTING	19

FIGURE 5 SYSTEM OPERATION BLOCK DIAGRAM………………………………………………………………………………………………………………..25

FIGURE 6 HARDWARE CONNECTION…………………………………………………………………………………………………………………………………..22

FIGURE 7 FINAL DESIGN……………………………………………………………………………………………………………………………………………………..30

LIST OF TABLES

Table 1 SCOPE OF STUDY	10

Table 2 RESEARCH DESIGN	13

Table 3 TRIAGE CLASSIFICATION ALGORITHMS	17

Table 4 HARDWARE COMPONENTS USED	21

CHAPTER ONE: INTRODUCTION

## 1.1 Background of the Study

Healthcare service delivery is a critical component of modern society. Hospitals often experience overcrowding, especially in outpatient departments where patients arrive seeking consultation and treatment. Manual queue management methods are prone to human error, inefficiency, and long waiting times.

Traditional hospital queue systems rely heavily on reception staff to issue tokens and guide patients. However, such methods cannot guarantee fairness, prioritization of emergency cases, or real-time communication with patients.

Embedded systems and Internet-of-Things inspired technologies have been widely adopted to automate service processes in healthcare environments. Queue management systems help hospitals organize patient flow, reduce congestion, and improve overall service quality.

This project proposes the development of a Smart Hospital Queue Management System using microcontroller technology. The system combines triage-based classification, automated token generation, and SMS notification functionality.

## 1.2 Problem Statement

Hospitals in many developing regions face significant challenges in managing outpatient queues. Patients are often required to wait for long periods without receiving updates about their service status. Emergency patients may not receive priority attention in manual systems.

The absence of automated communication systems contributes to congestion and patient dissatisfaction. Therefore, there is a need to develop a low-cost embedded queue management solution suitable for resource-constrained healthcare facilities.

## 1.3 Research Objectives

General Objective

To design and implement a microcontroller-based smart hospital queue management system with triage selection and SMS notification.

Specific Objectives

- To design a keypad-based token booking interface.

- To implement symptom-based triage classification.

- To develop queue tracking and display logic using LCD.

- To integrate GSM communication for SMS notification.

- To evaluate system performance through prototype testing.

## 1.4 Research Questions

- How can patient queue management be automated using embedded system technology?

- How effective is triage-based classification in outpatient service delivery?

- What is the performance impact of SMS notification in queue management?

## 1.5 Significance of the Study

The project contributes to healthcare service improvement by demonstrating how low-cost embedded systems can be used to automate outpatient queue control. The system benefits patients by reducing waiting time and improving information accessibility.

The study is important to healthcare institutions, embedded system developers, and researchers exploring smart healthcare technologies.

## 1.6 Scope of the Study

The project focuses on outpatient queue management using prototype hardware consisting of:

Table 1 SCOPE OF STUDY

| PROTOTYPE | FUNCTION |
| --- | --- |
| 1.Microcontroller | Control unit |
| 2.Keypad | Input interface |
| 3.LCD | Display unit |
| 4.buzzer | Alert system |
| 5.GSM | SMS notification module |

The system is not intended to replace full hospital management systems but to demonstrate embedded automation concepts.

## 1.7 Limitations of the Study

The project faced several limitations including:

- Dependence on GSM network availability

- Prototype-level implementation rather than full hospital deployment

- Power stability constraints during hardware testing

CHAPTER TWO: LITERATURE REVIEW

## 2.1 Introduction to Literature Review

Literature review provides theoretical and empirical foundation for the development of the Smart Hospital Queue Management System. Previous studies have explored the use of automation in healthcare service delivery, especially in patient flow control and communication systems, have explored the integration of digital token systems, electronic displays, and mobile notifications to manage patient flow.

## 2.2 Concept of Queue Management Systems

Queue management systems are designed to organize waiting lines and optimize service delivery. In healthcare environments, queue systems help reduce patient congestion and improve fairness in service allocation.

Several researchers have demonstrated that automated queue systems outperform manual reception-based queue control in terms of efficiency and reliability. Embedded electronic control systems have become popular due to their affordability and ease of implementation.

Platforms such as Arduino provide a convenient environment for developing prototype healthcare automation systems. Previous research has shown that Arduino-based systems can be effectively applied in monitoring devices, automated hospital equipment, and queue management applications.

## 2.3 Embedded Systems in Healthcare Applications

Embedded systems are computer-based control systems designed to perform dedicated functions. In healthcare technology, embedded systems are used in patient monitoring, appointment scheduling, and medical device automation.

Microcontroller platforms such as Arduino-based architectures have been widely adopted in academic prototype development due to their simplicity and open-source support.

## 2.4 Triage Systems in Healthcare

Triage is a medical prioritization method used to determine treatment order based on patient condition severity. Traditional triage is performed by trained medical personnel during emergency situations.

Automating triage selection in this project is achieved through keypad-based symptom category input. The system classifies patients into emergency, normal, or review categories. This approach is intended for prototype demonstration and not clinical diagnosis.

## 2.5 SMS Communication in Healthcare Systems

Short Message Service (SMS) technology provides a simple and reliable method of patient notification. SMS-based queue notification reduces patient crowding in hospital waiting areas.

The integration of GSM communication modules allows the system to send queue status alerts when the patient is approaching service position.

## 2.6 Review of Related Studies

Several studies have proposed automated hospital queue systems using microcontrollers and wireless communication technologies.

- Some researchers developed token issuance systems using keypad interfaces and display modules.

- Others implemented patient notification systems using GSM communication.

- Previous designs often lacked triage prioritization functionality.

## 2.7 Research Gap

Despite advances in healthcare automation, there remains limited integration of triage selection, queue monitoring, and SMS notification within a single embedded prototype suitable for low-resource hospital environments.

This project addresses the gap by designing a unified Smart Hospital Queue Management System combining these functionalities.

# CHAPTER THREE:METHODOLOGY

## 3.1 Introduction

This chapter describes the research methodology used in the design and development of the Smart Hospital Queue Management System with Triage and SMS Notification. The methodology outlines the research design, system architecture, hardware components, software development, data collection methods, and testing procedures used to implement the system.

The main objective of the methodology is to provide a clear explanation of how the system was designed, developed, implemented, and evaluated to ensure that it meets the intended purpose of improving hospital queue management.

## 3.2 Research Design

The research adopted an experimental and system development design approach. This approach was suitable because the project involved designing and building a working prototype that demonstrates how automation can improve hospital queue management.

The research design followed several stages which included:

Table 2 RESEARCH DESIGN

- Problem identification – Understanding the challenges associated with manual queue systems in hospitals.

- System design – Designing the architecture of the proposed smart queue management system.

- Hardware implementation – Integrating electronic components such as the Arduino microcontroller, keypad, LCD display, buzzer, and GSM module.

- Software development – Writing and testing Arduino programs to control the system operations.

- System testing and evaluation – Testing the system functionality to ensure proper performance.

- Data analysis and documentation – Evaluating system performance and documenting the results.

This structured approach ensured that the project was completed in a systematic and organized manner.

## 3.3 System Architecture

The Smart Hospital Queue Management System consists of both hardware and software components that work together to automate patient queue handling.

The system operates using the following workflow:

- A patient arrives at the hospital reception.

- The patient enters information through the keypad interface.

- The system evaluates the patient's condition through the triage selection process.

- A token number is generated automatically.

- The token number is displayed on the LCD screen.

- The patient receives an SMS notification indicating their queue number.

- The system alerts patients when they are two people away from their turn.

- A buzzer alert and LCD display notify the patient when it is their turn.

- The system indicates the room number where the patient should go.

This architecture ensures efficient management of patient flow and reduces overcrowding in waiting areas.

## 3.4 System Block Diagram

The system is composed of several interconnected modules which work together to achieve the queue management functionality.

Main system components include:

- Input Module (Keypad)

- Processing Unit (Arduino Microcontroller)

- Display Module (I2C LCD)

- Alert System (Buzzer)

- Communication Module (SIM800L GSM)

- Power Supply Unit

FIGURE 5 SYSTEM FLOW

System Flow

Patient Input → Arduino Processing → Token Generation → LCD Display → SMS Notification → Buzzer Alert → Room Direction

The microcontroller acts as the central control unit that processes inputs and controls all system outputs.

## 3.5 Hardware Components

The hardware components used in the project were selected based on;

- reliability,

- affordability, and

- ease of integration.

### 3.5.1 Arduino Uno Microcontroller

The Arduino Uno serves as the main controller of the system. It processes inputs from the keypad and controls the output devices such as the LCD display, buzzer, and GSM module.

Key features include:

- ATmega328P microcontroller

- 14 digital input/output pins

- 6 analog input pins

- USB interface for programming

- Operates at 5V

The Arduino was selected because it is easy to program and widely used in embedded systems development.

### 3.5.2 Matrix Keypad

A 3×4 matrix keypad was used as the input interface of the system. It allows patients to interact with the system and select their symptoms or request a token number.

Functions of the keypad include:

- Selecting triage options

- Booking tokens

- Navigating system options

Matrix keypads are efficient because they reduce the number of required input pins.

### 3.5.3 LCD Display (16×2 I2C)

The system uses a 16×2 LCD display with an I2C interface to display information to users.

The LCD shows:

- Token numbers

- Triage status

- Queue updates

- Room directions

The I2C interface reduces wiring complexity since it requires only two communication lines.

### 3.5.4 Buzzer

A buzzer is used as an audio alert system. It notifies patients when:

- They are close to their turn

- It is their turn to see the doctor

The buzzer improves accessibility for patients who may not be looking at the display.

### 3.5.5 SIM800L GSM Module

The SIM800L GSM module is responsible for sending SMS notifications to patients.

Functions include:

- Sending token confirmation messages

- Alerting patients when they are next in line

- Notifying patients of their consultation room

The GSM module communicates with the Arduino through serial communication using AT commands.

### 3.5.6 Power Supply Unit

The system requires a stable power supply for proper operation.

The following power sources were used:

- Arduino USB power supply

- Lithium battery for GSM module

- Capacitor for voltage stabilization

The capacitor helps prevent voltage drops that may occur during GSM transmission.

## 3.6 Software Development

The software for the system was developed using the Arduino Integrated Development Environment (Arduino IDE).

The programming language used was C++ for Arduino, which allows direct interaction with the microcontroller hardware.

The program includes the following functions:

- Keypad scanning

- Token generation

- Triage classification

- Queue management

- LCD display control

- Buzzer alert system

- SMS notification using GSM module

Libraries used in the program include:

- Keypad library

- LiquidCrystal_I2C library

- SoftwareSerial library

These libraries simplify communication with the hardware components.

## 3.7 Triage Classification Algorithm

The triage system allows patients to select their condition using keypad options.

The system classifies patients into three categories:

Table 3 TRIAGE CLASSIFICATION ALGORITHMS

CATEGORY FUNCTION

1. Emergency - Patients with severe symptoms are given priority and directed immediately to the emergency room.

2. Normal Consultation - Patients with regular medical conditions are placed in the normal queue.

3. Review Appointment - Patients visiting for follow-up consultations are assigned a separate queue.

The triage algorithm ensures that critical patients receive medical attention faster.

## 3.8 Data Collection

Data collection during the project focused on system performance and functionality.

The following data were recorded:

- Number of tokens generated

- Queue processing time

- SMS delivery success rate

- System response time

Testing was conducted using simulated patient inputs through the keypad.

## 3.9 System Testing

Several tests were conducted to evaluate the functionality of the system.

FIGURE 1 LCD AND KEYPAD TESTING

FIGURE 2 TOKEN GENEREATION TESTING

FIGURE 3 GSM TESTING

### 3.9.1 Hardware Testing

Each hardware component was tested individually before full system integration.

Examples include:

- Testing keypad input

- Testing LCD display messages

- Testing buzzer alerts

- Testing GSM SMS transmission

### 3.9.2 Integration Testing

Integration testing involved connecting all system components together and testing the overall system functionality.

The following operations were verified:

- Token generation

- Queue updates

- SMS notifications

- Alert system

### 3.9.3 Performance Testing

The system was evaluated based on:

- Speed of token generation

- SMS delivery time

- Accuracy of triage classification

- Reliability of hardware components

The system performed efficiently under simulated hospital conditions.

## 3.10 Ethical Considerations

The system does not store sensitive patient medical data. During testing, simulated data was used to ensure privacy and ethical compliance.

If implemented in real hospitals, the system would need to follow healthcare data protection policies.

## 3.11 Chapter Summary

This chapter presented the methodology used in designing and implementing the Smart Hospital Queue Management System. The chapter explained the research design, system architecture, hardware components, software development process, triage algorithm, and system testing procedures.

The next chapter presents the system design, implementation, and results of the developed system.

# CHAPTER FOUR

# SYSTEM DESIGN, IMPLEMENTATION AND RESULTS

## 4.1 Introduction

This chapter presents the detailed design, implementation, and operational results of the Smart Hospital Queue Management System with Triage and SMS Notification. The chapter explains how the system components were designed, connected, programmed, and tested to achieve the project objectives.

The system integrates hardware components and embedded software to automate hospital queue management. It allows patients to book tokens through a keypad, automatically classifies patients using a triage selection system, displays queue information on an LCD screen, alerts patients through a buzzer, and sends SMS notifications using a GSM module.

## 4.2 System Design Overview

The system was designed as a microcontroller-based embedded system consisting of input devices, a pro

cessing unit, output devices, and a communication module.

The system design focuses on the following key functions:

- Patient token booking

- Triage-based patient classification

- Queue monitoring and control

- Audio and visual alerts

- SMS notifications to patients

The Arduino Uno microcontroller acts as the central processing unit that coordinates communication between all modules.

## 4.3 Hardware Design

The hardware design involved connecting all electronic components in a way that allows them to communicate efficiently with the microcontroller.

### 4.3.1 Hardware Components Used

The following components were used in the development of the system:

Table 4 HARDWARE COMPONENTS USED

Component FUNCTION

Arduino Uno; System controller. 3×4 Matrix Keypad; Patient input interface. Buzzer; Audio alert system. SIM800L GSM Module; SMS communication. Lithium Battery; GSM power supply. Capacitor; Stabilizes GSM voltage.

Each component performs a specific role within the system.

## 4.4 Circuit Design

The circuit design involved connecting all hardware modules to the Arduino microcontroller.

Figure 6 HARDWARE CONNECTION

### 4.4.1 Keypad Connections

The 3×4 keypad consists of 4 rows and 3 columns.

Example connection:

| Keypad Pin | Arduino Pin |
| --- | --- |
| Row1 | Pin 9 |
| Row2 | Pin 8 |
| Row3 | Pin 7 |
| Row4 | Pin 6 |
| Column1 | Pin 5 |
| Column2 | Pin 4 |
| Column3 | Pin 3 |

The keypad allows patients to interact with the system by selecting triage options.

### 4.4.2 LCD I2C Connections

The LCD display uses the I2C communication protocol, which requires only two data lines.

| LCD Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

The LCD is used to display:

- Token number

- Queue updates

- Patient instructions

- Room directions

### 4.4.3 Buzzer Connection

The buzzer is connected to a digital output pin of the Arduino.

| Buzzer Pin | Arduino Pin |
| --- | --- |
| Positive | Pin 12 |
| Negative | GND |

The buzzer produces an audible sound when it is a patient's turn.

### 4.4.4 GSM Module Connections

The SIM800L GSM module communicates with Arduino through serial communication.

| GSM Pin | Arduino Pin |
| --- | --- |
| VCC | Lithium Battery (3.7–4.2V) |
| GND | Arduino GND |
| TX | Arduino Pin 7 |
| RX | Arduino Pin 8 (through voltage divider) |

A 10kΩ resistor voltage divider was used to reduce the Arduino 5V signal to a safe level for the GSM RX pin.

## 4.5 Software Design

The system software was developed using Arduino IDE. The program was written in Arduino C++.

The program performs the following operations:

- Initializes system components

- Reads keypad input

- Generates patient tokens

- Classifies patients using triage logic

- Displays queue information on LCD

- Sends SMS notifications using GSM module

- Activates buzzer alerts

The program runs continuously in a loop to monitor patient input and update the queue.

## 4.6 Token Generation System

The token generation system assigns a unique number to each patient when they book a consultation.

The process works as follows:

- Patient presses the token booking key.

- The system increments the token counter.

- The token number is displayed on the LCD screen.

- The patient receives an SMS confirmation message.

Example:

Patient 1 → Token 001
Patient 2 → Token 002
Patient 3 → Token 003

This automated process eliminates manual queue management.

## 4.7 Triage Selection System

The triage system allows patients to select the type of medical service they require.

The keypad options were designed as follows:

| Key | Function |
| --- | --- |
| 1 | Emergency |
| 2 | Normal Consultation |
| 3 | Review Appointment |

After selection, the system assigns the patient to the appropriate queue.

Example:

Emergency patient → Immediate priority
Normal patient → Standard queue
Review patient → Follow-up queue

This feature helps hospitals prioritize patients according to urgency.

## 4.8 SMS Notification System

The system uses the SIM800L GSM module to send SMS notifications to patients.

The SMS notifications include:

- Token confirmation

- Queue alerts

- Room direction

Example SMS:

"Your token number is 12. You are two patients away from your turn."

When the patient is next in line, the system sends another message:

"Token 12, please proceed to Room 3."

The GSM module uses AT commands to send the SMS messages.

Example commands used:

AT
AT+CMGF=1
AT+CMGS="+2547XXXXXXXX"

These commands enable the Arduino to communicate with the GSM network.

4.9 Buzzer Alert System

The buzzer provides audio alerts to ensure patients do not miss their turn.

The buzzer activates in two situations:

- When the patient is two people away

- When it is their turn

This ensures that patients remain attentive even if they are not watching the display.

## 4.10 System Operation

Figure 5 system operation block diagram

The full system operates according to the following sequence:

- The patient arrives at the reception.

- The patient selects their condition using the keypad.

- The system generates a token number.

- The LCD displays the token number.

- The GSM module sends an SMS confirmation.

- The queue advances as patients are served.

- When a patient is two people away, the system sends an alert.

- When it is their turn, the buzzer sounds and the LCD shows the room number.

This automated process improves efficiency in hospital waiting areas.

## 4.11 System Testing Results

The system was tested using simulated patient inputs.

The results were evaluated based on:

- Token generation accuracy

- Queue update speed

- SMS delivery

- Alert system functionality

Test Results

| Test Item | Result |
| --- | --- |
| Keypad Input | Successful |
| Token Generation | Accurate |
| LCD Display | Working correctly |
| Buzzer Alert | Functional |
| GSM SMS Notification | Successful |
| Triage Classification | Correct |

The system successfully performed all required functions.

## 4.12 Challenges Encountered

Several challenges were encountered during the project development.

GSM Communication Issues

During testing, the GSM module initially sent incorrect SMS characters due to communication errors between the Arduino and the GSM module.

This problem was addressed by:

- Adjusting the baud rate

- Stabilizing the power supply

- Adding capacitors

Keypad Hardware Issues

A faulty keypad caused some keys to fail during early testing. The issue was solved by replacing the keypad with a new unit.

Power Stability

The GSM module required a stable voltage supply. Voltage drops caused temporary communication failures.

Adding a capacitor and a dedicated lithium battery improved system stability.

## 4.13 System Advantages

The developed system provides several advantages:

- Reduces hospital congestion

- Improves patient experience

- Prioritizes emergency cases

- Automates queue management

- Provides real-time patient alerts

These benefits make the system suitable for hospitals with high patient traffic.

## 4.14 Chapter Summary

This chapter presented the design, implementation, and testing results of the Smart Hospital Queue Management System. The chapter explained how the hardware components were connected, how the software was developed, and how the system operates in a real hospital scenario.

The next chapter presents the conclusion and recommendations based on the results of the project.

# CHAPTER FIVE:

# CONCLUSION AND RECOMMENDATIONS

## 5.1 Introduction

This chapter presents the summary of the study, conclusions drawn from the project, and recommendations for future improvements of the Smart Hospital Queue Management System with Triage and SMS Notification. The chapter evaluates how the project objectives were achieved and discusses the potential impact of the system on hospital service delivery.

The project aimed to design and implement a low-cost automated queue management system that improves patient flow in hospitals by integrating token generation, triage classification, queue monitoring, and SMS notification technology.

## 5.2 Summary of the Study

Hospitals, particularly in developing countries, often face challenges related to long patient waiting times, disorganized queues, and inefficient communication between patients and healthcare providers. Traditional queue systems rely heavily on manual processes, which can lead to confusion, overcrowding, and patient dissatisfaction.

The purpose of this project was to develop a Smart Hospital Queue Management System that automates patient queue management using embedded system technology.

The project involved the design and implementation of a system consisting of several key components including:

- Arduino microcontroller

- 3×4 matrix keypad for patient input

- 16×2 I2C LCD display for queue information

- SIM800L GSM module for SMS notifications

- Buzzer for audio alerts

The system allows patients to book a queue token using a keypad, after which the system automatically assigns a token number and categorizes the patient based on a triage selection process.

The system then performs the following operations:

- Displays queue information on an LCD screen

- Notifies patients via SMS when they are two patients away from their turn

- Alerts the patient when it is their turn to see the doctor

- Directs the patient to the correct consultation room

The system was tested under simulated conditions to verify the functionality of all modules.

The results showed that the system was capable of:

- Generating patient tokens automatically

- Classifying patients using triage logic

- Displaying queue status correctly

- Sending SMS notifications to patients

- Providing audio alerts using a buzzer

The system therefore demonstrated its capability to improve queue management efficiency in hospital outpatient departments.

## 5.3 Achievement of Project Objectives

The objectives outlined at the beginning of the project were successfully achieved.

Figure 7 FINAL DESIGN

### Objective 1: Design a keypad-based token generation system

This objective was achieved by developing a system that allows patients to interact with a 3×4 keypad to book their consultation token. The Arduino microcontroller processes the keypad input and automatically generates the next available token number.

### Objective 2: Implement a triage-based patient classification system

The system includes a triage selection feature that allows patients to select their condition category using the keypad.

The categories include:

- Emergency

- Normal consultation

- Review appointment

This functionality allows hospitals to prioritize urgent medical cases.

### Objective 3: Display queue information on an LCD screen

The 16×2 I2C LCD display successfully provides real-time information to patients including:

- Token number

- Queue status

- Instructions to proceed to the consultation room

This ensures that patients remain informed about their position in the queue.

### Objective 4: Implement SMS notification using a GSM module

The SIM800L GSM module was integrated into the system to send SMS notifications to patients.

The system sends alerts when:

- The patient successfully books a token

- The patient is two people away from their turn

- It is the patient’s turn to see the doctor

This functionality reduces overcrowding in hospital waiting areas.

### Objective 5: Provide audio alerts for queue notifications

A buzzer alert system was successfully implemented to notify patients when their turn approaches.

The buzzer improves system accessibility by ensuring that patients who may not be watching the display can still receive alerts.

## 5.4 Conclusion

Based on the system design, implementation, and testing results, it can be concluded that the Smart Hospital Queue Management System with Triage and SMS Notification provides an effective solution for improving patient queue management in healthcare facilities.

The system successfully demonstrates how embedded systems and communication technologies can be used to automate hospital processes and enhance service delivery.

Key benefits of the system include:

- Reduction of patient waiting time

- Improved queue organization

- Prioritization of emergency cases

- Better communication with patients through SMS alerts

- Reduction of overcrowding in hospital waiting areas

The system is low-cost, scalable, and suitable for deployment in small and medium healthcare facilities, particularly in developing countries where advanced hospital management systems may not be affordable.

The successful development of this project demonstrates the practical application of mechatronic engineering principles in healthcare technology.

## 5.5 Recommendations

Although the system performed well during testing, several improvements can be made to enhance its functionality and real-world applicability.

Integration with Hospital Database

Future versions of the system can integrate with a hospital electronic medical record (EMR) database to automatically store patient information and consultation records.

Mobile Application Integration

A mobile application can be developed to allow patients to:

- Book appointments remotely

- Check queue status

- Receive notifications through the app

This would further reduce hospital congestion.

Touch Screen Interface

Replacing the keypad with a touch screen interface could improve user interaction and provide a more modern system design.

Multiple Department Support

The system can be expanded to support multiple hospital departments such as:

- Laboratory services

- Pharmacy queues

- Radiology departments

This would transform the system into a full hospital queue management platform.

Cloud-Based Monitoring

Future versions of the system can upload queue data to a cloud server, allowing hospital administrators to monitor patient flow remotely.

## 5.6 Suggestions for Further Research

Further research can be conducted in the following areas:

- Artificial intelligence–based triage classification

- IoT-based hospital management systems

- Integration of biometric patient identification

- Real-time analytics for hospital patient flow management

These advancements would significantly improve the efficiency and intelligence of hospital service delivery systems.

## 5.7 Final Remarks

The Smart Hospital Queue Management System developed in this project demonstrates the potential of embedded systems technology to address real-world healthcare challenges.

By combining microcontroller automation, triage logic, GSM communication, and real-time patient alerts, the system offers an innovative approach to improving hospital service efficiency.

This project therefore contributes to the growing field of smart healthcare systems and digital hospital management technologies.

# REFERENCE

Alhassan, A.R. & Asante, A.F. (2020). Improving healthcare service delivery through queue management systems.

Choudhary, D. & Shah, K. (2021). Smart hospital queue management using IoT and embedded systems.

Arduino Documentation (2024). Arduino Uno Technical Reference.

# APPENDIX 1:Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Keypad.h>

#include <SoftwareSerial.h>

LiquidCrystal_I2C lcd(0x27,16,2);

SoftwareSerial gsm(7,8);

int buzzer = 9;

// Queue arrays

int emergencyQueue[20];

int normalQueue[20];

int reviewQueue[20];

String phoneNumbers[60];

int eFront=0,eRear=0;

int nFront=0,nRear=0;

int rFront=0,rRear=0;

int token=1;

int servingToken=0;

String phone="";

int triageType=0;

// Keypad

const byte ROWS=4;

const byte COLS=3;

char keys[ROWS][COLS]={

{'1','2','3'},

{'4','5','6'},

{'7','8','9'},

{'*','0','#'}

};

byte rowPins[ROWS]={5,4,3,2};

byte colPins[COLS]={A2,A1,A0};

Keypad keypad=Keypad(makeKeymap(keys),rowPins,colPins,ROWS,COLS);

void setup()

{

Serial.begin(9600);

gsm.begin(9600);

pinMode(buzzer,OUTPUT);

lcd.init();

lcd.backlight();

lcd.print("MEDIQUEUE");

lcd.setCursor(0,1);

lcd.print("Starting...");

delay(3000);

showTriage();

Serial.println("==== MEDIQUEUE SYSTEM READY ====");

Serial.println("Press 'n' in Serial Monitor to call next patient");

}

void loop()

{

char key = keypad.getKey();

if(key)

{

// TRIAGE SELECTION

if(triageType==0)

{

if(key=='1'||key=='2'||key=='3')

{

triageType=key-'0';

lcd.clear();

lcd.print("Enter Phone:");

lcd.setCursor(0,1);

}

}

// PHONE ENTRY

else

{

if(key=='#')

{

registerPatient();

}

else if(key=='*')

{

phone="";

lcd.setCursor(0,1);

lcd.print("                ");

}

else

{

phone+=key;

lcd.setCursor(0,1);

lcd.print(phone);

}

}

}

// NURSE CALLS NEXT PATIENT

if(Serial.available())

{

char cmd=Serial.read();

if(cmd=='n')

{

serveNext();

}

}

}

// SHOW TRIAGE MENU

void showTriage()

{

lcd.clear();

lcd.print("1:Emerg 2:Norm");

lcd.setCursor(0,1);

lcd.print("3:Review");

}

// REGISTER PATIENT

void registerPatient()

{

phoneNumbers[token]=phone;

if(triageType==1)

{

emergencyQueue[eRear++]=token;

}

else if(triageType==2)

{

normalQueue[nRear++]=token;

}

else if(triageType==3)

{

reviewQueue[rRear++]=token;

}

Serial.print("Token ");

Serial.print(token);

Serial.print(" Registered | Phone:");

Serial.print(phone);

Serial.print(" | Triage:");

Serial.println(triageType);

lcd.clear();

lcd.print("Token:");

lcd.print(token);

tone(buzzer,1000,200);

// SEND TOKEN SMS

sendTokenSMS(phone,token);

token++;

phone="";

triageType=0;

delay(3000);

showTriage();

}

// SERVE NEXT PATIENT

void serveNext()

{

int currentToken=0;

if(eFront<eRear)

{

currentToken=emergencyQueue[eFront++];

}

else if(nFront<nRear)

{

currentToken=normalQueue[nFront++];

}

else if(rFront<rRear)

{

currentToken=reviewQueue[rFront++];

}

else

{

lcd.clear();

lcd.print("No Patients");

Serial.println("Queue Empty");

delay(2000);

showTriage();

return;

}

servingToken=currentToken;

Serial.print("Serving Token: ");

Serial.println(currentToken);

lcd.clear();

lcd.print("Serving:");

lcd.print(currentToken);

tone(buzzer,1500,500);

// SEND TURN SMS

sendTurnSMS(phoneNumbers[currentToken],currentToken);

// ALERT NEXT PATIENT

checkTwoAway();

}

// ALERT TWO PATIENTS BEFORE

void checkTwoAway()

{

int nextToken=0;

if(eFront<eRear)

{

nextToken=emergencyQueue[eFront];

}

else if(nFront<nRear)

{

nextToken=normalQueue[nFront];

}

else if(rFront<rRear)

{

nextToken=reviewQueue[rFront];

}

if(nextToken!=0)

{

Serial.print("Alert Token ");

Serial.println(nextToken);

sendAlertSMS(phoneNumbers[nextToken],nextToken);

}

}

// SEND TOKEN SMS

void sendTokenSMS(String number,int tokenNo)

{

gsm.println("AT");

delay(1000);

gsm.println("AT+CMGF=1");

delay(1000);

gsm.print("AT+CMGS=\"");

gsm.print(number);

gsm.println("\"");

delay(1000);

gsm.print("Hospital Token: ");

gsm.print(tokenNo);

gsm.println(". Please wait for your turn.");

delay(500);

gsm.write(26);

delay(5000);

}

// ALERT SMS

void sendAlertSMS(String number,int tokenNo)

{

gsm.println("AT");

delay(1000);

gsm.println("AT+CMGF=1");

delay(1000);

gsm.print("AT+CMGS=\"");

gsm.print(number);

gsm.println("\"");

delay(1000);

gsm.print("Token ");

gsm.print(tokenNo);

gsm.println(": You are next. Please get ready.");

delay(500);

gsm.write(26);

delay(5000);

}

// TURN SMS

void sendTurnSMS(String number,int tokenNo)

{

gsm.println("AT");

delay(1000);

gsm.println("AT+CMGF=1");

delay(1000);

gsm.print("AT+CMGS=\"");

gsm.print(number);

gsm.println("\"");

delay(1000);

gsm.print("Token ");

gsm.print(tokenNo);

gsm.println(": It is now your turn. Please proceed to the room.");

delay(500);

gsm.write(26);

delay(5000);

}